ChatterTale Prototype Manual Setup

List of materials:

| Male-male dupont cables | 1 Pack |
| --- | --- |
| Male-female dupont cables | 1 Pack |
| Arduino Uno or mega | 1 |
| Adafruit Musicmaker shield | 1 |
| Push buttons | 5 |
| 250 ohm resistors | 5 |
| Max 9841 microphone | 1 |
| Headphone jack | 1 |
| 12 v battery | 1 |
| 10 k potentiometer | 2 |
| SD card | 1 |
| 3.5 mm Earphone jack | 1 |

Material for the amplifier:

| 10 k potentiometer | 1 |
| --- | --- |
| LM386 | 1 (This is a sensitive component: use caution) |
| 1.2k ½ watt | 1 |
| 10 nano farad ceramic capacitor | 1 (for the bypass) |
| 10 micro farad ceramic capacitor | 1 |
| 10 ohm ½ watt resistor | 1 |
| ≈ 0.05 micro farad ceramic capacitor | 1 |
| 250 micro farad electrolytic capacitor | 1 |

- Take an Arduino Uno board and a Adafruit Musicmaker MP3 shield and attach them together from the pins as shown. Start by placing the MP3 shield on top of the Arduino board.

- Next, apply the push buttons to the breadboard. Take the breadboard and 5 push buttons and 5 male-to-female wires. You will also need 5 250 ohm 0.25 W resistors. Follow the diagram below when assigning the wires and resistors on the breadboard (Note the resistors in the diagram don’t match the resistors we need, diagram is for placement guidance). Connect the wires from the breadboard to the digital pins of the MP3 shield (digital pins # 9,8,5 and 2). The last wire will be pinned to the RESET header on the shield. Pin 2 will be for Pause/Play. Pin 5 will be fast forward/skip forward. Pin 8 will be record. Pin 9 will be rewind/skip backward. And finally the reset pin will restart the device.

- Next, place a potentiometer on the same breadboard where you previously placed the push buttons. The potentiometer is going to serve as a volume dial.  If it doesn't already have 3 wires soldered to it, do so. With the dial facing towards you, take the leftmost wire and pin it to the plus column in your breadboard. Then take the rightmost wire and pin it to the minus column on your breadboard. Now with the middle wire, pin it directly to the breadboard and take a male to female wire and pin it to the breadboard next to the middle wire you pinned from the potentiometer and pin it to the MP3 shield on Analog 0 “A0”.

- Place a LED light indicator onto the breadboard. Looking at the LED's pin ends, there should be one long end and one  short end. Taking the short end, place it into the minus column of your breadboard, then proceed to placing the long end pin on your breadboard.  Next take a resistor as well as another male to female wire and follow the diagram to place the resistor and the wire so that they are aligned with the LED light placed on the breadboard. The other end of the wire should be pinned to Digital pin 10 on the surface of the MP3 shield.

- Next take microphone component along with 1 male-to-female wire and 2 male-to-male wires. Follow the diagram below: with the “Out” pin wire a male-to-female wire that from the breadboard to the surface of the music shield to the M+ pin. With the next pin on the microphone, “VDD” take a male-to-male wire and connect the “VDD” pin to the minus column on the breadboard. Finally, use a male-to-male wire to connect the “GND” pin on the microphone wire with the plus column on the breadboard.

- Using 2 male-to-female wires connect one on the plus column from the breadboard to the “5V” pin on surface of the MP3 shield. The second wire will be connected from the minus column of the breadboard to the “GND” pin on the surface of the MP3 shield as detailed below

- The next step is to setup the audio switchboard. Connect the “GND” pin on the MP3 shield with the minus column of your second breadboard. Using a second wire, connect the MP3 shield pin “Vin” and the plus column of the new breadboard.

- Next, using two male-to-male wires connect the two sides of the breadboard: plus to plus and minus to minus as shown.

- The next step is to install the audio amplifier. This is chip labeled “LM386” and is very sensitive to electrostatic discharge. It is recommended to use  Before placing it onto the breadboard take a look to see if there is a groove on the amplifier. Use this notch to orientate the chip so that the notch is on the left side of the amplifier.  Use the diagram as a guide for placement.

- Take a 1.2K ohm resistor and using the diagram as reference, take one end of the pin and place it next to the far bottom left pin of the amplifier. Then take a 10uF (micro farads) capacitor that is 25V and place the pin end that doesn’t have white shaded on the capacitor and place it next to the top far left pin of your amplifier.  All that is left is to place the other end pin of the resistor and the capacitor next to each other as shown on the diagram.

- Connect a 100nF (nano farad) capacitor to pin 7 of the amplifier as well as to ground (the minus column of the breadboard).

- Connect pin number 6 of the amplifier to the plus column of the breadboard with a male-to-male wire.

- Next, with the potentiometer in hand, and with the dial facing towards you take the left wire and place it in the minus column of your breadboard.

- Pin the middle wire of the potentiometer to pin 3 of the amplifier.

- The headphone jack has 5 leads, 1 in the front and 4 in the back. Connect the back 4 leads to the breadboard to 4 different terminal strips with the headphone jack facing away from you.

- Connect the headphone aux cable by taking the green and purple wires and pining them aligned to the far left wire of the headphone jack.  Then with the yellow wire from the headphone aux cord, pin it next to the far right wire of the headphone jack.

- Back to the potentiometer. Connect the mid-left wire of the headphone jack to the far right terminal of the potentiometer.

- Plug the headphone aux cable into the headphone jack on the Musicmaker shield.

(Note: Green is right output, Yellow is left output and Purple is GND)

- Connect pin 4 of the amplifier to the minus column of the breadboard.

- Next we need to take our 250uF capacitor next to the top far right pin of the amplifier. Be sure that the pin is the one on the white shaded side of the capacitor. Then with the other pin place it into the breadboard. We then need to take our speaker and look for the minus polarity wire from our speaker and pin it in front of the capacitor we had just placed. (The pin that isn’t next to the amplifier.) Going back to the speaker, place the other pin into the minus column of our breadboard. Next taking our resistor place one end of the pin behind the capacitor we had just placed and the other endpin to the right of it.  Next we need to take are last 2 1uF capacitors and place them behind the right pin of our resistor we just placed. Be sure that the resistors are aligned in a diagonally parallel pacement and that the white shaded sides of the capacitors are facing to the left. (Refer to the diagram) Finally we must place a male to male wire next to our 2 capacitors and pin the other end to the minus column of the bread board.

- Now that we have our setup for testing, it’s time to format our Arduino component. To start you will need a micro SD card  the capacity of the SD card depends on the user . Load into the SD card some music or audio files. Be sure that they are in MP3 format. (Ex. Music.mp3) IMPORTANT. The file name must be equal or less than 8 characters or the device won’t play the files.  (Ex. abcdefgh.mp3) Rename the file if need on a computer before you upload them onto the SD card also don´t forget to add the file named v44k1q05.img that is a patch from adafruit that makes work the recording that patch can be found at: https://github.com/adafruit/Adafruit_VS1053_Library/blob/master/examples/record_ogg/v44k1q05.img

Once you have your formatted micro SD card place the card into the SD slot on the surface of the MP3 shield

- Next we need to upload our program into the Arduino board. We need to have the Arduino program environment to do it, and can be founded at: http://arduino.cc/en/Main/Software the Arduino development tool you can upload the program from there. Start by connecting your Arduino directly to the computer. You will need a C2G USB cable in order to do this. Once you have your cable connected, go back to your Arduino development tool on your computer and in the overhead toolbar select Tools and then select Board, you should be using an Arduino. Then go to Serial Port under Tools and select usbmodem. Now that we have configured our upload settings we can proceed to upload our program. Looking back at the Arduino development tool on our computer click on the arrow to upload the program onto the Arduino. You should get a window prompt that says upload complete when the upload is finished. If this does not work at the beginning or if your computer doesn’t ‘recognize the Arduino you must go to the device manager (win 7, vista, xp) and you will find something like this:

Then to install the drivers:

And then we will be ready to start uploading the program.

- Then you will need some libraries that we already upload into the github page, the link is the following https://github.com/rao102/AudioProject/tree/master/libraries

Download the zip file and then extract it on C:\Users\yourcomputername\Documents\Arduino\libraries after that you will  only have to load the program and press verify and upload and its done, but don´t forget to go to: https://github.com/rao102/Audio-Project/blob/master/150218_final_uno.ino

Just copy the code and paste it into the Arduino programming environment, make sure you copy all the code otherwise is not going to compile, but if the program is ready you will see something like this:

And when you upload the program you will see something like this:

- Finally we need to set up our power input. We actually need to set two input power sources. One that will be used for power up the buttons, other for power up  the Arduino and the other power output that will go directly onto the audio switch board this last one should be at least 12v DC. Start with a 12v DC rechargeable battery that has a output cord and an input cord but if you have a charger of 12 volts that can work as well. We also are going to need a 12V 9V DC power Y Splitter Cable with one male head output cord cut off with 2 jumper wires soldered to it. Start with plugging the Y cable into the battery, then proceed to pin the plus and minus polarities of the two jumper wires into the audio switch breadboard in the plus and minus columns respectively. All that is left is to plus the other Y male cord into the Arduino and hit the power switch on the battery. Now your playback set up is ready. Test the buttons and have fun!

If you need to view a full diagram for this setup follow the link below:

https://raw.githubusercontent.com/rao102/Audio-Project/master/resources/chattertale_bb.png

Block Diagram

Troubleshooting